#### Muhammad Salman

#### IU05-0322-0038

#### Assignment-3

#### QUESTION 1)

#### Digital Clock with 7-Segment Display

- Scenario: Build a digital clock that displays hours and minutes (HH:MM).

- Task: Interface the AVR with four common-cathode 7-segment displays (using multiplexing for display efficiency). Use a Timer configured to generate a $1\text{ Hz}$ interrupt to increment the seconds counter, which then updates the minutes and hours.

CODE :

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

volatile uint8_t seconds = 0;

volatile uint8_t minutes = 30;

volatile uint8_t hours = 12;

const uint8_t SEGMENT_MAP[] = {

0x3F,

0x06,

0x5B,

0x4F,

0x66,

0x6D,

0x7D,

0x07,

0x7F,

0x6F

};

void timer1_init()

{

TCCR1B |= (1 << WGM12);

TCCR1B |= (1 << CS12);

OCR1A = 31249;

TIMSK |= (1 << OCIE1A);

}

ISR(TIMER1_COMPA_vect)

{

seconds++;

if (seconds >= 60)

{

seconds = 0;

minutes++;

if (minutes >= 60)

{

minutes = 0;

hours++;

if (hours >= 24)

{

hours = 0;

}

}

}

}

void display_digit(uint8_t digit_pos, uint8_t value)

{

PORTD = 0xFF;

PORTC = SEGMENT_MAP[value];

PORTD &= ~(1 << digit_pos);

_delay_ms(5);

}

int main(void)

{

DDRC = 0xFF;

DDRD = 0x0F;

MCUCSR |= (1<<JTD);

MCUCSR |= (1<<JTD);

timer1_init();

sei();

while(1)

{

uint8_t min_ones = minutes % 10;

uint8_t min_tens = minutes / 10;

uint8_t hr_ones  = hours % 10;

uint8_t hr_tens  = hours / 10;

display_digit(0, min_ones);

display_digit(1, min_tens);

display_digit(2, hr_ones);

display_digit(3, hr_tens);

}

}

#### QUESTION 2)

#### Doorbell Simulator with Persistent Ring

- Scenario: A doorbell system where a brief press results in a continuous ring for 3 seconds.

- Task: Use a push button input. When pressed, an output LED (simulating the bell) turns $\text{ON}$ and stays $\text{ON}$ for exactly 3 seconds, then turns $\text{OFF}$.

- Concepts: Timing control (_delay_ms() or simple Timer), Edge detection.

Code :

#define F_CPU 8000000UL // 8 MHz Clock

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRC |= (1 << PC0);  // Set PC0 as Output

PORTC &= ~(1 << PC0); // Initially turn LED OFF

DDRA &= ~(1 << PA0); // Set PA0 as Input

PORTA |= (1 << PA0); // Enable Internal Pull-up Resistor on PA0

while(1)

{

if (!(PINA & (1 << PA0))) // Agar PA0 Low (0)

{

_delay_ms(50);

if (!(PINA & (1 << PA0)))

{

PORTC |= (1 << PC0);

_delay_ms(3000);

PORTC &= ~(1 << PC0);

}

}

}

}

#### QUESTION 3)

#### Automatic Pet Feeder Timer

- Scenario: Design a system that dispenses pet food exactly once every 12 hours.

- Task: Use a Timer to track time. When 12 hours have elapsed, activate an output pin (simulating a servo motor or solenoid) for $500\text{ ms}$ to release food, then reset the timer.

- Concepts: Timer/Counter for long time delays, Fixed interval timing.

Code :

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#define FEED_INTERVAL_SECONDS 5

volatile uint32_t seconds_counter = 0;

volatile uint8_t feed_flag = 0;

---

void timer1_init()

{

TCCR1B |= (1 << WGM12);

TCCR1B |= (1 << CS12);

OCR1A = 31249;

TIMSK |= (1 << OCIE1A);

}

ISR(TIMER1_COMPA_vect)

{

seconds_counter++;

if (seconds_counter >= FEED_INTERVAL_SECONDS)

{

feed_flag = 1;

seconds_counter = 0;

}

}

int main(void)

{

DDRC |= (1 << PC0);

PORTC &= ~(1 << PC0);

timer1_init();

sei();

while(1)

{

if (feed_flag == 1)

{

PORTC |= (1 << PC0);

_delay_ms(500);

PORTC &= ~(1 << PC0);

feed_flag = 0;

}

}

}